# NUMERICAL STUDY OF FLOW AND HEAT TRANSFER IN A STRAIGHT DUCT CONTAINING A CIRCULAR REGION WITH A PAIR OF FINS

Selma Akçay1*, Nasser Abdoul-Halim Ismael2

1Çankırı Karatekin University, Engineering Faculty, Department of Mechanical Engineering, Çankırı, Türkiye

2Çankırı Karatekin University, Undergraduate Student of Mechanical Engineering, Çankırı, Türkiye

*1Orcid ID: 0000-0003-2654-0702

*Corresponding author: selmaakcay@karatekin.edu.tr

ABSTRACT Wavy channels are widely used in many heat transfer devices, especially heat exchangers [1]. Experimental and numerical studies have reported that higher heat transfer rates are achieved in wavy channels compared to straight channels due to the higher heat transfer surface area [2-3]. It is known that fins added into the channel increase thermal enhancement [4-5]. However, wavy channels and fins slightly increase the pressure drop [6]. In this study, a large circular region in a straight duct is considered in order to reduce the pressure drop. Fins are used to improve flow mixing. This study numerically investigates flow and heat transfer characteristics in a straight duct containing a circular region with a pair of fins. Analyzes are carried out with ANSYS Fluent solver. The pressure-velocity connection is handled with the SIMPLE algorithm. There are adiabatic straight sections at the inlet and outlet of the duct. The duct structure contains a circular region near the inlet of the duct, and a pair of fins are installed within the circular region. The walls of the duct before and after the circular region are flat. The circular region and subsequent channel surfaces are kept at a constant temperature of Ts=350K. Nusselt number (Nu), thermal enhancement factor (TEF), pressure drop, friction factor (f), and performance factor (PF) are calculated for different Reynolds numbers. The results of the study are given as a function of dimensionless numbers. The numerical study is compared with previous study results [7]. To observe the effects of the circular region and the pair of fins on the flow and temperature fields, velocity and temperature contours are obtained, and the results are discussed. In addition, the study results are compared to the duct without fins and the straight duct. Numerical results show that the fins in the circular region increased the Nusselt number. However, the presence of fins causes a slight increase in pressure drop. References: [1] Alam T, & Kim MH. (2018). A comprehensive review on single phase heat transfer enhancement techniques in heat exchanger applications. Renewable and Sustainable Energy Reviews, 81, 813-839. [2] Kurtulmus N, Sahin B. (2019). A review of hydrodynamics and heat transfer through corrugated channels. International Communications in Heat and Mass Transfer, 108, 104307. [3] Ajarostaghi SSM, Zaboli M, Javadi H, Badenes B, & Urchueguia JF. (2022). A review of recent passive heat transfer enhancement methods. Energies, 15, 986. [4] Modi, Ashish J., & Manish K. Rathod. (2019). Comparative study of heat transfer enhancement and pressure drop for fin-and-circular tube compact heat exchangers with sinusoidal wavy and elliptical curved rectangular winglet vortex. International Journal of Heat and Mass Transfer, 141, 310-326. [5] Nitturi LK, Kapu VK, Gugulothu R, Kaleru A, Vuyyuri V, & Farid A. (2023). Augmentation of heat transfer through passive techniques. Heat Transfer, https://doi.org/10.1002/htj.22877 [6] Akçay S. (2023). Numerical analysis of hydraulic and thermal performance of Al2O3-water nanofluid in a zigzag channel with central winglets. Gazi University Journal of Science, 36 (1), 383-397. [7] Wang G, Qia C, Liu M, Li C, Yan Yu, & Liang L. (2019). Effect of corrugation pitch on thermo-hydraulic performance of nanofluids in corrugated tubes of heat exchanger system based on exergy efficiency. Energy Conversion and Management; 186, 51–65.

# Keywords: Circular region, Fins, Straight duct, Heat transfer, Laminar flow, Thermal enhancement